Branch Office Configuration

All Commands Used — Config, Troubleshooting & Fixes

Every command typed during the branch office build, firewall integration and final fix — in the order it happened.

This document captures every configuration and troubleshooting command used during the branch office build. It is split into sections matching the build sequence — R2 setup, switch configuration, OSPF, DHCP, firewall attempts, and the final working configuration. Commands are shown with plain-English explanations of what they do and why they were used.

# Section 1 — R2 Initial Configuration

R2 is the branch office router. It connects to R1 via a WAN link and to the branch switches via its LAN interface. These are the commands used to bring R2 up from scratch.

## 1.1 R2 Interface Configuration

! Enter privileged and config mode

R2> enable

R2# configure terminal

! Set hostname so it appears in the prompt and SSH key

R2(config)# hostname R2

! WAN interface facing R1 — point-to-point /30 subnet

R2(config)# interface gigabitEthernet0/0

R2(config-if)# ip address 10.0.0.2 255.255.255.252

R2(config-if)# no shutdown

R2(config-if)# exit

! LAN interface facing SW5 — branch gateway

R2(config)# interface gigabitEthernet0/1

R2(config-if)# ip address 192.168.40.1 255.255.255.0

R2(config-if)# no shutdown

R2(config-if)# exit

! Loopback for stable OSPF Router ID — never goes down

R2(config)# interface loopback 0

R2(config-if)# ip address 2.2.2.2 255.255.255.255

R2(config-if)# exit

R2(config)# end

R2# write memory

WHY /30 ON WAN  A /30 subnet gives exactly 2 usable host IPs — one for each end of the point-to-point link between routers. No other devices live on this segment so a larger subnet wastes addresses.

## 1.2 R2 Verification Commands

R2# show ip interface brief

! Confirms G0/0, G0/1 and Loopback0 are all up/up

R2# show interfaces g0/0

! Shows detailed stats — confirm internet address matches 10.0.0.2/30

R2# show interfaces g0/1

! Confirm 192.168.40.1/24 on LAN side

# Section 2 — SW5 and SW6 Configuration

SW5 is the branch core switch connecting R2 to SW6. SW6 is the branch access switch connecting the branch PCs and server. This section was revisited multiple times during troubleshooting.

## 2.1 SW5 Configuration

! Create VLAN 40 for the branch department

SW5# configure terminal

SW5(config)# vlan 40

SW5(config-vlan)# name Branch

SW5(config-vlan)# exit

! Gig0/1 faces R2 — access port on VLAN 40 (R2 has no subinterfaces)

SW5(config)# interface gig0/1

SW5(config-if)# switchport mode access

SW5(config-if)# switchport access vlan 40

SW5(config-if)# exit

! Fa0/1 faces SW6 — trunk to carry VLAN 40 between switches

SW5(config)# interface fa0/1

SW5(config-if)# switchport mode trunk

SW5(config-if)# exit

SW5(config)# end

SW5# write memory

ACCESS vs TRUNK on SW5  Gig0/1 facing R2 is an ACCESS port because R2 has a plain IP address with no subinterfaces — it cannot handle 802.1Q tagged frames. Fa0/1 facing SW6 is a TRUNK because SW6 needs to carry VLAN 40 frames between the two switches.

## 2.2 SW6 Configuration

! Create VLAN 40 — must exist on every switch in the path

SW6# configure terminal

SW6(config)# vlan 40

SW6(config-vlan)# name Branch

SW6(config-vlan)# exit

! Fa0/1 faces SW5 — trunk

SW6(config)# interface fa0/1

SW6(config-if)# switchport mode trunk

SW6(config-if)# exit

! PC-facing ports — access on VLAN 40

SW6(config)# interface fa0/2

SW6(config-if)# switchport mode access

SW6(config-if)# switchport access vlan 40

SW6(config-if)# exit

SW6(config)# interface fa0/3

SW6(config-if)# switchport mode access

SW6(config-if)# switchport access vlan 40

SW6(config-if)# exit

SW6(config)# interface fa0/4

SW6(config-if)# switchport mode access

SW6(config-if)# switchport access vlan 40

SW6(config-if)# exit

SW6(config)# interface fa0/5

SW6(config-if)# switchport mode access

SW6(config-if)# switchport access vlan 40

SW6(config-if)# exit

SW6(config)# end

SW6# write memory

## 2.3 Switch Verification Commands

SW5# show vlan brief

! VLAN 40 must be active with Gig0/1 listed under it

SW5# show interfaces trunk

! Fa0/1 must show trunking with VLAN 40 active

SW5# show interfaces gig0/1 switchport

! Must show: Administrative Mode: static access, Access Mode VLAN: 40

SW5# show cdp neighbors

! Shows which devices are physically connected to SW5 and on which port

SW6# show mac address-table

! Shows which MAC addresses are learned on which ports — confirms PCs are on correct ports

# Section 3 — OSPF Configuration

OSPF was configured on both R1 and R2 so that routes are shared dynamically. R1 advertises its default route to R2 so branch devices can reach the internet. This section was the most troublesome — the network statement had to be corrected multiple times.

## 3.1 OSPF on R1

R1# configure terminal

R1(config)# router ospf 1

! Process ID 1 — local only, does not need to match R2

R1(config-router)# router-id 1.1.1.1

! Manually set Router ID using loopback IP — more stable than auto-selection

R1(config-router)# network 192.168.10.0 0.0.0.255 area 0

! Advertise IT VLAN — wildcard 0.0.0.255 = match entire /24 subnet

R1(config-router)# network 192.168.20.0 0.0.0.255 area 0

R1(config-router)# network 192.168.30.0 0.0.0.255 area 0

R1(config-router)# network 10.0.0.0 0.0.0.3 area 0

! Advertise WAN link — wildcard 0.0.0.3 = /30 subnet

R1(config-router)# default-information originate

! Sends R1s default route (toward ISP) to R2 via OSPF — critical for branch internet access

R1(config-router)# exit

R1(config)# end

R1# write memory

## 3.2 OSPF on R2

R2# configure terminal

R2(config)# router ospf 1

R2(config-router)# router-id 2.2.2.2

R2(config-router)# network 192.168.40.0 0.0.0.255 area 0

! Advertise branch subnet

R2(config-router)# network 10.0.0.0 0.0.0.3 area 0

! Advertise WAN link — must match actual interface subnet

R2(config-router)# exit

R2(config)# end

R2# write memory

## 3.3 OSPF Troubleshooting Commands Used

R1# show ip ospf neighbor

! Must show R2 (2.2.2.2) with state FULL/DR or FULL/BDR

R2# show ip ospf neighbor

! Must show R1 (1.1.1.1) with state FULL

R2# show ip route

! Look for O routes (OSPF learned) for HQ subnets and O*E2 for default route

R1# show running-config | section ospf

! Verify exact network statements — this caught the 10.10.10.0 vs 10.0.0.0 mismatch

R2# show running-config | section ospf

! Same check on R2 side

## 3.4 OSPF Fix Commands — Correcting Wrong Network Statements

! Remove wrong statement — always include area 0 or command is incomplete

R1(config-router)# no network 10.10.10.0 0.0.0.3 area 0

! Add correct statement matching actual interface IP

R1(config-router)# network 10.0.0.0 0.0.0.3 area 0

! Remove duplicate OSPF process that was accidentally created

R2(config)# no router ospf 2

! Then re-add missing network into the correct single process

R2(config)# router ospf 1

R2(config-router)# network 192.168.40.0 0.0.0.255 area 0

R2(config-router)# network 10.0.0.0 0.0.0.3 area 0

OSPF LESSON  The network statement subnet must exactly match the actual interface IP. Always run show ip interface brief before writing OSPF network statements to confirm the exact subnet. Missing 'area 0' at the end causes 'incomplete command' error.

# Section 4 — DHCP Configuration on R2

R2 serves as the DHCP server for the branch VLAN 40. Branch PCs get their IPs directly from R2 since R2 is their gateway.

## 4.1 DHCP Pool Configuration

! Exclude gateway and server IPs before creating pool

R2(config)# ip dhcp excluded-address 192.168.40.1 192.168.40.9

R2(config)# ip dhcp pool BRANCH_POOL

R2(config-dhcp)# network 192.168.40.0 255.255.255.0

! Defines which subnet this pool serves

R2(config-dhcp)# default-router 192.168.40.1

! Gateway that PCs will use — must be R2s own interface IP

R2(config-dhcp)# dns-server 8.8.8.8

R2(config-dhcp)# exit

R2(config)# end

R2# write memory

## 4.2 DHCP Verification Commands

R2# show ip dhcp binding

! Lists every leased IP, which MAC received it, and expiry time

R2# show ip dhcp pool

! Shows total, leased and excluded address counts for the pool

R2# show running-config | section dhcp

! Confirms pool config and excluded addresses are saved correctly

# Section 5 — Branch ACL Security Policy on R1

The branch security policy states that branch PCs can reach HQ IT only — they cannot access HR or Sales. The ACL is applied on R1 because all branch traffic passes through R1 to reach HQ departments. The existing PROTECT_HR and PROTECT_SALES ACLs were updated to include branch deny rules.

## 5.1 Rebuilding ACLs with Branch Deny Rules

! Delete existing ACL entirely and rebuild with correct rule order

R1(config)# no ip access-list extended PROTECT_HR

R1(config)# ip access-list extended PROTECT_HR

! Rule 10 — block Sales from HR

R1(config-ext-nacl)# 10 deny ip 192.168.30.0 0.0.0.255 192.168.20.0 0.0.0.255

! Rule 20 — block Branch from HR

R1(config-ext-nacl)# 20 deny ip 192.168.40.0 0.0.0.255 192.168.20.0 0.0.0.255

! Rule 30 — allow IT to reach HR (admin access)

R1(config-ext-nacl)# 30 permit ip 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255

! Rule 40 — allow everything else (internet, other permitted traffic)

R1(config-ext-nacl)# 40 permit ip any any

R1(config-ext-nacl)# exit

! Rebuild PROTECT_SALES with branch deny

R1(config)# no ip access-list extended PROTECT_SALES

R1(config)# ip access-list extended PROTECT_SALES

R1(config-ext-nacl)# 10 deny ip 192.168.20.0 0.0.0.255 192.168.30.0 0.0.0.255

R1(config-ext-nacl)# 20 deny ip 192.168.40.0 0.0.0.255 192.168.30.0 0.0.0.255

R1(config-ext-nacl)# 30 permit ip any any

R1(config-ext-nacl)# exit

R1(config)# end

R1# write memory

ACL ORDER LESSON  Deny rules MUST appear before permit ip any any. The router stops at the first match — if permit any any is above a deny rule, the deny is never reached. Always rebuild ACLs from scratch when inserting new deny rules to ensure correct ordering.

## 5.2 ACL Verification

R1# show ip access-lists

! Shows all ACL rules with hit counters — confirm branch deny rules are above permit any any

! Hit count increments every time a packet matches that rule

# Section 6 — NAT Update for Branch Subnet

R1's NAT ACL originally only covered the three HQ subnets. The branch subnet 192.168.40.0/24 needed to be added so branch PCs can reach the internet through R1's NAT.

## 6.1 Adding Branch to NAT ACL

R1# configure terminal

R1(config)# ip access-list standard NAT_TRAFFIC

R1(config-std-nacl)# permit 192.168.40.0 0.0.0.255

! Add branch subnet to the NAT selector ACL

R1(config-std-nacl)# exit

R1(config)# end

R1# write memory

## 6.2 Critical Fix — ip nat inside on R1 G0/2

This was the final fix that got branch internet working. R1's G0/2 interface facing R2 was not marked as ip nat inside. Branch traffic arrived at R1 via G0/2 but was not being translated because the interface was not in the NAT inside zone.

R1# configure terminal

R1(config)# interface g0/2

R1(config-if)# ip nat inside

! Every interface through which inside traffic flows must be marked nat inside

R1(config-if)# exit

R1(config)# end

R1# write memory

CRITICAL NAT LESSON  Every router interface through which internal traffic flows must be marked 'ip nat inside' — including WAN links between routers. Missing this on G0/2 meant branch traffic reached R1 but was never translated, causing internet to fail silently.

## 6.3 NAT Verification Commands

R1# show ip access-lists NAT_TRAFFIC

! Confirm 192.168.40.0 permit is in the list

R1# show ip nat translations

! After pinging from branch PC — should show translation entries for 192.168.40.x

R1# show ip nat statistics

! Shows total translations and which interfaces are inside/outside

R1# show ip interface g0/2

! Look for: Outgoing access list... and NAT: inside — confirms nat inside is applied

# Section 7 — ASA Firewall Configuration Attempts

This section documents every command attempted during the Branch-FW configuration. These commands are kept for reference — some worked, some revealed Packet Tracer ASA limitations. The firewall was ultimately removed from the branch traffic path.

## 7.1 ASA 5506-X Attempts (first firewall model used)

! Basic interface config — nameif never properly activated in Packet Tracer

HQ-FW(config)# interface g1/1

HQ-FW(config-if)# nameif outside

HQ-FW(config-if)# security-level 0

HQ-FW(config-if)# ip address 10.2.2.2 255.255.255.252

HQ-FW(config-if)# no shutdown

HQ-FW(config)# interface g1/2

HQ-FW(config-if)# nameif inside

HQ-FW(config-if)# security-level 100

HQ-FW(config-if)# ip address 192.168.40.1 255.255.255.0

HQ-FW(config-if)# no shutdown

! Default route toward R2

HQ-FW(config)# route outside 0.0.0.0 0.0.0.0 10.2.2.1

! ACL to permit all traffic — testing purposes

HQ-FW(config)# access-list PERMIT_ALL extended permit ip any any

HQ-FW(config)# access-list PERMIT_ALL extended permit icmp any any

HQ-FW(config)# access-group PERMIT_ALL in interface outside

HQ-FW(config)# access-group PERMIT_ALL in interface inside

## 7.2 ASA 5505 Attempts (replacement model)

! ASA 5505 uses VLAN-based interfaces — different from 5506-X

Branch-FW(config)# interface vlan 1

Branch-FW(config-if)# nameif inside

Branch-FW(config-if)# security-level 100

Branch-FW(config-if)# ip address 192.168.40.1 255.255.255.0

Branch-FW(config-if)# no shutdown

Branch-FW(config)# interface vlan 2

Branch-FW(config-if)# nameif outside

Branch-FW(config-if)# security-level 0

Branch-FW(config-if)# ip address 10.2.2.2 255.255.255.252

Branch-FW(config-if)# no shutdown

! Map physical ports to VLANs

Branch-FW(config)# interface ethernet0/0

Branch-FW(config-if)# switchport access vlan 2

Branch-FW(config-if)# no shutdown

Branch-FW(config)# interface ethernet0/1

Branch-FW(config-if)# switchport access vlan 1

Branch-FW(config-if)# no shutdown

! Default route

Branch-FW(config)# route outside 0.0.0.0 0.0.0.0 10.2.2.1

## 7.3 DHCP on ASA — Commands That Were Accepted

! Address pool — space-separated range NOT accepted, use dash

Branch-FW(config)# dhcpd address 192.168.40.10-192.168.40.41 inside

! Warning: limited to 32 addresses — .41 used instead of .50

Branch-FW(config)# dhcpd dns 8.8.8.8 interface inside

! dhcpd enable inside — FAILED: need to define address pool range first

! Root cause: pool and enable must be configured in same session without gaps

## 7.4 Commands That Failed — Packet Tracer ASA Limitations

| Command attempted | Error received | What it means |
| --- | --- | --- |
| nat (inside,outside) dynamic interface | Invalid input | Modern NAT syntax not supported on this ASA version in Packet Tracer |
| dhcprelay server 10.2.2.1 | Unrecognised | DHCP relay not supported on this ASA model in Packet Tracer |
| ip helper-address 10.2.2.1 | Invalid input | IOS command — does not work on ASA CLI |
| same-security-traffic permit inter-interface | Invalid input | Not supported in this Packet Tracer ASA version |
| fixup protocol icmp | Unrecognised | Legacy ASA 7.x command — not supported in version 8.6+ |
| show nameif | Invalid input | Must be: show interface followed by interface name |
| policy-map + inspect icmp | class-map not configured | inspection_default class-map must exist first — not pre-configured in Packet Tracer |

## 7.5 ASA Verification Commands That Did Work

Branch-FW# show run

! Shows complete running configuration including interface IPs and ACLs

Branch-FW# show ip address

! ASA-style command showing IP addresses on all named interfaces

Branch-FW# show route

! Routing table — confirm default route via outside interface

Branch-FW# show access-list

! All ACLs with hit counters

Branch-FW# show run | include access-group

! Shows which ACLs are applied to which interfaces

Branch-FW# show arp

! ARP table showing which IPs are reachable on inside/outside

Branch-FW# ping 10.2.2.1

! Ping from ASA CLI to R2 — confirmed outside interface working

Branch-FW# ping 192.168.40.10

! Ping from ASA CLI to branch PC — confirmed inside interface working after IP was added

Branch-FW# show conn

! Active connections tracked by stateful firewall

Branch-FW# show dhcpd state

! Confirms whether DHCP server is active on inside interface

# Section 8 — Final Working Configuration

After removing Branch-FW from the traffic path and restoring direct R2 to SW5 connection, these are the final commands that got everything working. This is the clean configuration to rebuild from.

## 8.1 R2 — Final Complete Configuration

! Interfaces

R2(config)# interface g0/0

R2(config-if)# ip address 10.0.0.2 255.255.255.252

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# interface g0/1

R2(config-if)# ip address 192.168.40.1 255.255.255.0

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# interface loopback 0

R2(config-if)# ip address 2.2.2.2 255.255.255.255

R2(config-if)# exit

! OSPF

R2(config)# router ospf 1

R2(config-router)# router-id 2.2.2.2

R2(config-router)# network 192.168.40.0 0.0.0.255 area 0

R2(config-router)# network 10.0.0.0 0.0.0.3 area 0

R2(config-router)# exit

! DHCP

R2(config)# ip dhcp excluded-address 192.168.40.1 192.168.40.9

R2(config)# ip dhcp pool BRANCH_POOL

R2(config-dhcp)# network 192.168.40.0 255.255.255.0

R2(config-dhcp)# default-router 192.168.40.1

R2(config-dhcp)# dns-server 8.8.8.8

R2(config-dhcp)# exit

! SSH

R2(config)# ip domain-name techcorp.com

R2(config)# crypto key generate rsa

! Enter 1024 when prompted

R2(config)# ip ssh version 2

R2(config)# username admin privilege 15 secret Cisco123

R2(config)# line vty 0 4

R2(config-line)# login local

R2(config-line)# transport input ssh

R2(config-line)# exit

R2(config)# end

R2# write memory

## 8.2 R1 — Branch-Related Additions to Existing Config

! G0/2 — WAN facing R2 — must be nat inside

R1(config)# interface g0/2

R1(config-if)# ip address 10.0.0.1 255.255.255.252

R1(config-if)# ip nat inside

R1(config-if)# no shutdown

R1(config-if)# exit

! Add branch subnet to NAT ACL

R1(config)# ip access-list standard NAT_TRAFFIC

R1(config-std-nacl)# permit 192.168.40.0 0.0.0.255

R1(config-std-nacl)# exit

! OSPF additions for WAN link and default route advertisement

R1(config)# router ospf 1

R1(config-router)# network 10.0.0.0 0.0.0.3 area 0

R1(config-router)# default-information originate

R1(config-router)# exit

R1(config)# end

R1# write memory

## 8.3 SW5 — Final Configuration

SW5(config)# vlan 40

SW5(config-vlan)# name Branch

SW5(config-vlan)# exit

SW5(config)# interface gig0/1

SW5(config-if)# switchport mode access

SW5(config-if)# switchport access vlan 40

SW5(config-if)# exit

SW5(config)# interface fa0/1

SW5(config-if)# switchport mode trunk

SW5(config-if)# exit

SW5(config)# end

SW5# write memory

## 8.4 SW6 — Final Configuration

SW6(config)# vlan 40

SW6(config-vlan)# name Branch

SW6(config-vlan)# exit

SW6(config)# interface fa0/1

SW6(config-if)# switchport mode trunk

SW6(config-if)# exit

SW6(config)# interface fa0/2

SW6(config-if)# switchport mode access

SW6(config-if)# switchport access vlan 40

SW6(config-if)# exit

SW6(config)# interface fa0/3

SW6(config-if)# switchport mode access

SW6(config-if)# switchport access vlan 40

SW6(config-if)# exit

SW6(config)# interface fa0/4

SW6(config-if)# switchport mode access

SW6(config-if)# switchport access vlan 40

SW6(config-if)# exit

SW6(config)# end

SW6# write memory

# Section 9 — Full Branch Verification Sequence

Run these commands in this exact order to confirm the branch is fully working. Start from Layer 1 and work up.

## Step 1 — Physical layer

SW5# show cdp neighbors

! Confirm R2 visible on Gig0/1 and SW6 visible on Fa0/1

SW6# show cdp neighbors

! Confirm SW5 visible on Fa0/1

## Step 2 — Layer 2 VLANs

SW5# show vlan brief

! VLAN 40 Branch active — Gig0/1 listed under it

SW6# show vlan brief

! VLAN 40 Branch active — Fa0/2, Fa0/3, Fa0/4 listed

SW5# show interfaces trunk

! Fa0/1 trunking with VLAN 40 active

SW6# show interfaces trunk

! Fa0/1 trunking with VLAN 40 active

## Step 3 — Layer 3 routing

R2# show ip interface brief

! G0/0 and G0/1 must show up/up

R2# show ip ospf neighbor

! R1 (1.1.1.1) must show FULL/DR or FULL/BDR

R2# show ip route

! Must show O routes for HQ subnets and O*E2 0.0.0.0/0 default route

## Step 4 — DHCP

R2# show ip dhcp binding

! Branch PCs should show here with 192.168.40.10+ addresses

## Step 5 — NAT

R1# show ip access-lists NAT_TRAFFIC

! Must include permit 192.168.40.0

R1# show ip nat translations

! After pinging from branch PC — entries for 192.168.40.x should appear

## Step 6 — End-to-end ping test from BR-PC1

ping 192.168.40.1

! R2 gateway — confirms DHCP and basic Layer 3

ping 192.168.10.10

! HQ IT-PC1 — confirms OSPF routing is working

ping 8.8.8.8

! Internet — confirms NAT is working end-to-end

ping 192.168.20.10

! HR-PC1 — should FAIL (blocked by PROTECT_HR ACL on R1)

TechCorp Branch Office — Complete Command Reference  |  April 2026

Every command that was tried, what worked, what failed, and the final working config.